SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Dr.V.Parthipan

Annexure A

Debugging & Optimisation Task

Code of Conduct:

I Yaswanth R(192572023) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Yaswanth R

Annexure A

Debugging & Optimisation Task

- A rapidly growing IT company has shifted its operations to a newly constructed six-storey smart office building. More than 1,200 employees use wireless laptops, smartphones, IP phones, printers, and IoT devices connected through the company's Wireless Local Area Network (WLAN). The office relies heavily on cloud computing, online meetings, video conferencing, and file sharing, making uninterrupted wireless connectivity essential for daily operations. In addition, the company has a team of field engineers and sales executives who travel across different cities and use 4G/5G Wireless Wide Area Network (WWAN) services to access the company's servers, customer databases, and business applications from remote locations.

During the last few weeks, employees have reported several connectivity issues. Users experience slow internet speeds during peak working hours, frequent Wi-Fi disconnections while moving between floors, and weak wireless signals in conference rooms. Video conferencing applications often freeze or disconnect, affecting communication with international clients. Network administrators have also observed that some wireless access points are overloaded while others remain underutilized, leading to uneven traffic distribution. Furthermore, field engineers working in rural and highway areas frequently lose WWAN connectivity, delaying the upload of maintenance reports and customer information. Security audits reveal that a few wireless devices are still connected using outdated security protocols, exposing sensitive company information to potential cyber threats. The management has requested the network engineering team to investigate these issues, identify the root causes, and optimize both the WLAN and WWAN infrastructure to ensure high performance, secure communication, and seamless connectivity for all users.

- A multinational financial organization operates its headquarters, regional offices, and multiple data centers using an Asynchronous Transfer Mode (ATM) network to support mission-critical services such as online banking, video conferencing, Voice over IP (VoIP), and real-time financial transactions. The ATM network uses fixed-size 53-byte cells and establishes Virtual Circuits (VCs) to provide reliable and predictable Quality of Service (QoS). Recently, the organization has experienced several performance issues during peak business hours. Customers report delays in online transactions, voice calls suffer from jitter and occasional packet loss, and video conferencing sessions experience reduced quality. Network monitoring indicates that several ATM switches are operating at high utilization, some Virtual Circuits are congested, and available bandwidth is not being efficiently allocated among different traffic classes. Engineers also discover that certain applications are using inappropriate ATM service categories, leading to increased delay and inefficient resource utilization. The IT department has assigned you as the network engineer to investigate the ATM network, identify the faults, debug the performance issues, and recommend optimization strategies to restore efficient, reliable, and high-quality communication across the enterprise.

- A multinational e-commerce company has interconnected its headquarters, regional offices, warehouses, and cloud data centers using a Multiprotocol Label Switching (MPLS) network to ensure fast, secure, and reliable communication. The MPLS infrastructure supports critical business applications, including online order processing, enterprise resource planning (ERP), video conferencing, Voice over IP (VoIP), cloud-based inventory management, and real-time customer support. Recently, the organization has experienced frequent performance issues during peak business hours. Employees report slow access to cloud applications, delayed order processing, poor-quality voice calls, and interruptions during video conferences. Network monitoring reveals that some Label Switched Paths (LSPs) are heavily congested, while other paths remain underutilized. The IT team also discovers that several routers have incorrect label mappings, causing inefficient routing and increased latency. In addition, traffic engineering has not been properly configured, resulting in uneven bandwidth utilization across the MPLS backbone. Since these issues are affecting customer satisfaction and business operations, the network administrator has asked you to investigate the MPLS network, identify the faults, debug the performance problems, and recommend suitable optimization techniques to improve the overall efficiency and reliability of the network.

- A multinational logistics company operates an automated warehouse where barcode scanners, RFID readers, autonomous robots, and inventory management systems continuously exchange data over a computer network. Every day, thousands of shipment records, customer details, and inventory updates are transmitted between warehouse terminals and the central server. Recently, the company has observed that some shipment records are arriving with incorrect information, barcode scans occasionally fail, and inventory quantities in the database do not always match the physical stock. During peak hours, network monitoring reports an increase in transmission errors caused by electrical interference, signal attenuation, and network congestion. Although some corrupted frames are detected and retransmitted, others remain undetected, resulting in incorrect database entries and delayed order processing. The IT department suspects that the current error detection and correction mechanisms are not configured efficiently. As the network engineer, you are assigned to investigate the transmission errors, identify the weaknesses in the existing error control techniques, and recommend suitable debugging and optimization strategies to improve the reliability and accuracy of data communication.

- A leading online education platform conducts live classes and online examinations for more than 15,000 students across different locations. During examinations, the central server simultaneously sends question papers, images, videos, and answer submission acknowledgements to all students over the network. The server is equipped with a high-speed 10 Gbps connection, whereas many students access the system using slower broadband or mobile internet connections. During peak hours, students experience incomplete downloads, delayed page loading, duplicate packets, and repeated retransmissions. Some students receive the next set of data before their devices finish processing the previous one, causing receiver buffer overflow and application crashes. Network administrators also observe an excessive number of retransmissions, increased latency, reduced throughput, and poor bandwidth utilization. The current flow control mechanism is not effectively regulating the transmission rate between the sender and receivers. As the network engineer, you have been assigned to investigate the communication problems, identify the causes of inefficient flow control, debug the existing system, and recommend suitable optimization techniques to ensure reliable and efficient data transmission during high-traffic periods.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |

- A rapidly growing IT company has shifted its operations to a newly constructed six-storey smart office building. More than 1,200 employees use wireless laptops, smartphones, IP phones, printers, and IoT devices connected through the company's Wireless Local Area Network (WLAN). The office relies heavily on cloud computing, online meetings, video conferencing, and file sharing, making uninterrupted wireless connectivity essential for daily operations. In addition, the company has a team of field engineers and sales executives who travel across different cities and use 4G/5G Wireless Wide Area Network (WWAN) services to access the company's servers, customer databases, and business applications from remote locations.

During the last few weeks, employees have reported several connectivity issues. Users experience slow internet speeds during peak working hours, frequent Wi-Fi disconnections while moving between floors, and weak wireless signals in conference rooms. Video conferencing applications often freeze or disconnect, affecting communication with international clients. Network administrators have also observed that some wireless access points are overloaded while others remain underutilized, leading to uneven traffic distribution. Furthermore, field engineers working in rural and highway areas frequently lose WWAN connectivity, delaying the upload of maintenance reports and customer information. Security audits reveal that a few wireless devices are still connected using outdated security protocols, exposing sensitive company information to potential cyber threats. The management has requested the network engineering team to investigate these issues, identify the root causes, and optimize both the WLAN and WWAN infrastructure to ensure high performance, secure communication, and seamless connectivity for all users.

Answer : 1

WLAN and WWAN Case Study – Smart Office Building

- Root Causes of the Problems

| Problem | Root Cause |
| --- | --- |
| Slow internet during peak hours | Too many users connected to the same Access Point (AP), causing network congestion. |
| Wi-Fi disconnects while moving between floors | Poor roaming configuration and insufficient overlap between APs. |
| Weak signal in conference rooms | Improper AP placement, walls, glass, and furniture blocking signals. |
| Video conferencing freezes | High network traffic, low bandwidth, interference, and packet loss. |
| Some APs overloaded | Uneven distribution of users (load imbalance). |
| WWAN signal loss in rural/highway areas | Weak 4G/5G tower coverage and poor signal strength. |
| Security risks | Devices using old protocols like WEP/WPA instead of WPA3. |

- Network Diagram

INTERNET

|

+-------------+

| Firewall    |

+-------------+

|

+---------------+

| Core Router   |

+---------------+

|

+-----------------+

| Core Switch     |

+-----------------+

________________________________________________

|        |        |        |        |            |

AP1      AP2      AP3      AP4      AP5        AP6

(Floor1) (Floor2) (Floor3) (Floor4) (Floor5)  (Floor6)

|        |        |        |        |         |

Laptops   Smartphones IP Phones Printers IoT Devices

||

4G / 5G Network

||

Cellular Tower (WWAN)

||

Field Engineers & Sales Executives

(Remote Access to Company Server)

- Solutions to Improve WLAN

A. Reduce Network Congestion

- Install additional Access Points.

- Upgrade to Wi-Fi 6 or Wi-Fi 6E.

- Increase internet bandwidth.

B. Improve Roaming

- Configure seamless roaming (802.11r, 802.11k, 802.11v).

- Ensure AP signal overlap of about 15–20%

C. Improve Signal Strength

- Reposition APs in conference rooms.

- Remove obstacles where possible.

- Use dual-band (2.4 GHz + 5 GHz).

D. Load Balancing

- Enable AP Load Balancing.

- Distribute users evenly among nearby APs.

E. Improve Video Conferencing

- Configure Quality of Service (QoS).

- Give higher priority to Zoom, Microsoft Teams, and Google Meet traffic.

4. Solutions to Improve WWAN

- Use SIM cards from providers with better rural coverage.

- Enable automatic switching between 4G and 5G.

- Use signal boosters or external antennas in weak coverage areas.

- Allow offline data storage and automatic synchronization when the network is restored.

5. Security Improvements

- Upgrade all devices to WPA3.

- Disable WEP and WPA.

- Use strong passwords.

- Enable 802.1X Authentication (RADIUS Server).

- Use VPN for remote employees.

- Perform regular security audits and firmware updates.

- Optimized Network Architecture

INTERNET

|

Firewall + VPN

|

Core Router

|

Core Switch

|

-----------------------------------------

|       |       |       |       |       |

AP1     AP2     AP3     AP4     AP5     AP6

|         |       |       |       |       |

Balanced Users with Seamless Wi-Fi Roaming

WPA3 + QoS + Load Balancing

|

4G / 5G Cellular Network

7. Benefits After Optimization

- ✔ Faster internet speed during peak hours.

- ✔ Smooth Wi-Fi roaming between floors.

- ✔ Strong wireless signal in conference rooms.

- ✔ Stable video conferencing without interruptions.

- ✔ Balanced user traffic across Access Points.

- ✔ Better 4G/5G connectivity for field engineers.

- ✔ Secure communication using WPA3 and VPN.

- ✔ Improved productivity and reliable access to cloud services.

- A multinational financial organization operates its headquarters, regional offices, and multiple data centers using an Asynchronous Transfer Mode (ATM) network to support mission-critical services such as online banking, video conferencing, Voice over IP (VoIP), and real-time financial transactions. The ATM network uses fixed-size 53-byte cells and establishes Virtual Circuits (VCs) to provide reliable and predictable Quality of Service (QoS). Recently, the organization has experienced several performance issues during peak business hours. Customers report delays in online transactions, voice calls suffer from jitter and occasional packet loss, and video conferencing sessions experience reduced quality. Network monitoring indicates that several ATM switches are operating at high utilization, some Virtual Circuits are congested, and available bandwidth is not being efficiently allocated among different traffic classes. Engineers also discover that certain applications are using inappropriate ATM service categories, leading to increased delay and inefficient resource utilization. The IT department has assigned you as the network engineer to investigate the ATM network, identify the faults, debug the performance issues, and recommend optimization strategies to restore efficient, reliable, and high-quality communication across the enterprise

Answer : 2

ATM Network Performance Analysis, Debugging, and Optimizatio

ATM Network Diagram

┌─────────────────────────┐

│     Headquarters (HQ)   │

│ Online Banking Servers  │

│ VoIP & Video Conference │

└───────────┬─────────────┘

│

ATM Virtual Circuit (VC)

│

┌─────────▼─────────┐

│    ATM Switch 1   │

│ (High Utilization)│

└───────┬───────────┘

│

┌──────────────┼──────────────┐

│              │              │

ATM VC │       ATM VC │       ATM VC │

│              │              │

┌────────▼──────┐ ┌─────▼─────┐ ┌──────▼──────┐

│ ATM Switch 2  │ │ ATM Switch│ │ ATM Switch 4│

│ (Congested VC)│ │     3     │ │             │

└──────┬────────┘ └─────┬─────┘ └──────┬──────┘

│                │              │

┌───────▼───────┐ ┌──────▼──────┐ ┌─────▼────────┐

│ Regional Office│ │ Data Center │ │ Regional    │

│ Online Banking │ │ Database    │ │ Office      │

│ VoIP & Video   │ │ Transactions│ │ Customers   │

└────────────────┘ └─────────────┘ └─────────────┘

Problems:

✖ High ATM Switch Utilization

✖ Congested Virtual Circuits

✖ Wrong ATM Service Categories

✖ Bandwidth Inefficiency

Faults Identified

1. ATM Switch Congestion

- ATM switches are overloaded during peak hours.

- Cell queues become full.

- Results in delay and packet loss.

2. Congested Virtual Circuits (VCs)

- Multiple applications share the same VC.

- Causes heavy traffic.

- Leads to increased latency.

3. Incorrect ATM Service Categories

Applications are using unsuitable QoS classes.

Example:

| Application | Wrong Service | Correct Service |
| --- | --- | --- |
| VoIP | UBR | CBR |
| Video Conference | ABR | VBR |
| Banking Transactions | UBR | ABR |

4. Poor Bandwidth Allocation

Available bandwidth is not distributed according to application priority.

Result:

- Critical banking traffic waits.

- Voice and video quality decreases.

Performance Issues

Online Banking

- Slow transaction processing

- Delayed account updates

VoIP

- Voice jitter

- Packet loss

- Echo

Video Conferencing

- Video freezing

- Low quality

- Audio-video synchronization problems

Debugging Steps

Step 1

Monitor ATM switch utilization.

Check:

- CPU utilization

- Buffer occupancy

- Cell loss ratio

Step 2

Inspect Virtual Circuits.

Find:

- Congested VCs

- Overloaded paths

- Cell delay

Step 3

Verify QoS Service Categories.

Ensure:

- CBR → Voice

- VBR → Video

- ABR → Banking Transactions

- UBR → Email/File Transfer

Step 4

Measure Performance

Monitor:

- Cell Loss Ratio

- Delay

- Jitter

- Throughput

- Bandwidth utilization

Optimization Strategies

1. Assign Correct ATM Service Categories

| Traffic | ATM Service |
| --- | --- |
| Voice over IP | CBR |
| Video Conference | VBR |
| Banking Transactions | ABR |
| Email/File Transfer | UBR |

2. Load Balancing

- Distribute traffic across multiple VCs.

- Prevent one VC from becoming congested.

3. Increase ATM Switch Capacity

Upgrade:

- Switch hardware

- Buffer memory

- Processing capability

4. Bandwidth Reservation

Reserve dedicated bandwidth for:

- Online Banking

- VoIP

- Video conferencing

Lower-priority applications use remaining bandwidth.

5. Traffic Shaping

Control traffic entering the ATM network.

Benefits:

- Prevents sudden bursts

- Reduces congestion

- Improves QoS

6. Continuous Monitoring

Monitor:

- Switch utilization

- VC status

- QoS performance

- Cell loss

- Delay

Final Optimized ATM Network

Headquarters

│

Dedicated ATM VC

│

┌────────────────┐

│ ATM Switch (QoS│

│  & Load Balance)│

└───┬─────┬──────┘

│     │

CBR VC  │     │  VBR VC

(Voice)    │     │ (Video)

│     │

┌────▼──┐ ┌▼──────┐

│ ABR VC│ │UBR VC │

│Banking│ │ Email │

└───┬───┘ └──┬────┘

│         │

Regional Offices/Data Centers

✓ Correct QoS

✓ Efficient Bandwidth Allocation

✓ Reduced Congestion

✓ Low Delay

✓ Low Jitter

✓ High Reliability

Expected Results After Optimization

- Faster online banking transactions.

- Clear and stable VoIP calls with minimal jitter.

- High-quality video conferencing.

- Reduced ATM switch congestion.

- Efficient bandwidth utilization.

- Improved Quality of Service (QoS).

- Reliable communication across headquarters, regional offices, and data centers.

- A multinational e-commerce company has interconnected its headquarters, regional offices, warehouses, and cloud data centers using a Multiprotocol Label Switching (MPLS) network to ensure fast, secure, and reliable communication. The MPLS infrastructure supports critical business applications, including online order processing, enterprise resource planning (ERP), video conferencing, Voice over IP (VoIP), cloud-based inventory management, and real-time customer support. Recently, the organization has experienced frequent performance issues during peak business hours. Employees report slow access to cloud applications, delayed order processing, poor-quality voice calls, and interruptions during video conferences. Network monitoring reveals that some Label Switched Paths (LSPs) are heavily congested, while other paths remain underutilized. The IT team also discovers that several routers have incorrect label mappings, causing inefficient routing and increased latency. In addition, traffic engineering has not been properly configured, resulting in uneven bandwidth utilization across the MPLS backbone. Since these issues are affecting customer satisfaction and business operations, the network administrator has asked you to investigate the MPLS network, identify the faults, debug the performance problems, and recommend suitable optimization techniques to improve the overall efficiency and reliability of the network.

Answer : 3

MPLS Network Performance Analysis, Debugging, and Optimization

MPLS Network Diagram

┌────────────────────────────┐

│      Headquarters (HQ)     │

│ ERP | Order Processing     │

│ VoIP | Video Conference    │

└─────────────┬──────────────┘

│

MPLS Backbone

│

┌────────────▼────────────┐

│        MPLS Router      │

│ (Incorrect Label Mapping)│

└───────┬────────┬────────┘

│        │

LSP-1  │        │  LSP-2

(Congested)│        │(Underutilized)

│        │

┌────────────▼───┐ ┌──▼─────────────┐

│ MPLS Router    │ │ MPLS Router    │

│ Regional Office│ │ Cloud Data Ctr │

└──────┬─────────┘ └─────┬──────────┘

│                 │

┌───────▼───────┐ ┌──────▼─────────┐

│ Warehouses    │ │ Customer Support│

│ Inventory     │ │ VoIP & CRM      │

└───────────────┘ └─────────────────┘

Problems:

✖ Congested Label Switched Paths (LSPs)

✖ Incorrect Label Mapping

✖ Poor Traffic Engineering

✖ Uneven Bandwidth Utilization

Faults Identified

1. Congested Label Switched Paths (LSPs)

- Some LSPs carry excessive traffic.

- Other LSPs remain underutilized.

- Causes high delay and congestion.

2. Incorrect Label Mapping

- MPLS routers assign wrong labels.

- Packets follow inefficient paths.

- Results in increased latency.

3. Poor Traffic Engineering

- Traffic is not evenly distributed.

- Bandwidth is wasted on idle paths.

- Critical applications compete for limited bandwidth.

4. Uneven Bandwidth Utilization

- One path is overloaded.

- Other available paths are unused.

- Network resources are not efficiently utilized.

Performance Issues

Online Order Processing

- Slow order confirmation

- Delayed payment processing

ERP System

- Slow database synchronization

- Delayed reports

VoIP

- Voice delay

- Jitter

- Packet loss

Video Conferencing

- Video freezing

- Poor audio quality

- Call interruptions

Cloud Applications

- Slow access

- Increased response time

Debugging Steps

Step 1: Check MPLS Routers

Inspect:

- CPU utilization

- Memory usage

- Interface errors

Step 2: Verify Label Mappings

Check:

- Incoming labels

- Outgoing labels

- Label Switching Table (LFIB)

Correct any incorrect label assignments.

Step 3: Monitor Label Switched Paths (LSPs)

Identify:

- Congested LSPs

- Idle LSPs

- Packet delay

- Packet loss

Step 4: Analyze Bandwidth Usage

Measure:

- Throughput

- Link utilization

- Queue length

- Traffic load

Step 5: Verify QoS Configuration

Prioritize traffic:

- VoIP → Highest Priority

- Video Conference → High Priority

- ERP → Medium Priority

- Inventory Management → Medium Priority

- File Downloads → Low Priority

Optimization Techniques

1. Correct Label Mapping

- Configure proper MPLS labels.

- Ensure shortest and most efficient forwarding path.

- Reduce routing delay.

2. Implement Traffic Engineering (MPLS-TE)

- Redistribute traffic across multiple LSPs.

- Avoid congestion.

- Utilize all available bandwidth.

3. Load Balancing

- Balance traffic among multiple LSPs.

- Prevent a single path from becoming overloaded.

4. Configure QoS

Prioritize business-critical traffic.

| Application | Priority |
| --- | --- |
| VoIP | Highest |
| Video Conferencing | High |
| Order Processing | High |
| ERP | Medium |
| Inventory Management | Medium |
| File Transfer | Low |

5. Bandwidth Reservation

Reserve dedicated bandwidth for:

- Online orders

- VoIP

- Video conferencing

Remaining bandwidth can be used by less critical applications.

6. Continuous Monitoring

Regularly monitor:

- Router utilization

- LSP performance

- Label tables

- Bandwidth usage

- Delay and packet loss

Optimized MPLS Network

Headquarters

│

MPLS Backbone

│

┌─────────────────────┐

│ MPLS Core Router    │

│ Correct Labels      │

│ QoS + MPLS-TE       │

└───┬─────────┬────────┘

│         │

LSP-1│         │LSP-2

(Balanced)       (Balanced)

│         │

┌──────▼───┐ ┌───▼────────┐

│ Regional │ │ Cloud Data │

│ Office   │ │ Center     │

└──────┬───┘ └────┬───────┘

│           │

┌───────▼──────┐ ┌──▼──────────┐

│ Warehouses   │ │ Customer    │

│ Inventory    │ │ Support     │

└──────────────┘ └─────────────┘

✓ Correct Label Mapping

✓ Traffic Engineering Enabled

✓ Load Balanced LSPs

✓ QoS Prioritization

✓ Efficient Bandwidth Usage

✓ Low Latency

✓ High Reliability

Expected Results After Optimization

- Faster online order processing.

- Quick access to ERP and cloud applications.

- Clear VoIP calls with minimal jitter.

- Smooth, high-quality video conferencing.

- Balanced traffic across all Label Switched Paths.

- Reduced latency and packet loss.

- Efficient bandwidth utilization.

- A multinational logistics company operates an automated warehouse where barcode scanners, RFID readers, autonomous robots, and inventory management systems continuously exchange data over a computer network. Every day, thousands of shipment records, customer details, and inventory updates are transmitted between warehouse terminals and the central server. Recently, the company has observed that some shipment records are arriving with incorrect information, barcode scans occasionally fail, and inventory quantities in the database do not always match the physical stock. During peak hours, network monitoring reports an increase in transmission errors caused by electrical interference, signal attenuation, and network congestion. Although some corrupted frames are detected and retransmitted, others remain undetected, resulting in incorrect database entries and delayed order processing. The IT department suspects that the current error detection and correction mechanisms are not configured efficiently. As the network engineer, you are assigned to investigate the transmission errors, identify the weaknesses in the existing error control techniques, and recommend suitable debugging and optimization strategies to improve the reliability and accuracy of data communication.

Answer : 4

Error Detection and Correction in Warehouse Network

Network Diagram

Automated Warehouse Network

Barcode Scanner      RFID Reader      Warehouse Terminal

│                  │                    │

└──────────┬───────┴───────────┬────────┘

│                   │

┌─────▼───────────────────▼─────┐

│          Network Switch       │

│ (Electrical Noise/Congestion) │

└──────────────┬────────────────┘

│

Data Frames Transmitted

│

┌───────▼────────┐

│ Central Server │

│ Inventory DB   │

└───────┬────────┘

│

Inventory Reports

Problems:

✖ Transmission Errors

✖ Corrupted Frames

✖ Barcode Scan Failures

✖ Incorrect Inventory Records

✖ Delayed Order Processing

Faults Identified

1. Electrical Interference

- Electrical noise corrupts transmitted data.

- Frames may contain incorrect bits.

2. Signal Attenuation

- Signal strength decreases over long distances.

- Data may be lost or corrupted.

3. Network Congestion

- Heavy traffic during peak hours.

- Causes packet delay and retransmissions.

4. Weak Error Detection

- Some corrupted frames are detected.

- Others pass through without detection.

5. Inefficient Error Correction

- Some damaged frames are retransmitted.

- Undetected errors create incorrect database entries.

Performance Issues

Barcode Scanners

- Barcode reading failures

- Incorrect shipment information

RFID Readers

- Missing tag information

- Delayed inventory updates

Inventory Management

- Wrong stock quantity

- Database mismatch

Customer Orders

- Delayed order processing

Common Error Detection Techniques

| Technique | Purpose |
| --- | --- |
| Parity Bit | Detects single-bit errors |
| Checksum | Detects transmission errors |
| CRC (Cyclic Redundancy Check) | Detects burst errors (Most Reliable) |

Common Error Correction Techniques

| Technique | Purpose |
| --- | --- |
| ARQ (Automatic Repeat Request) | Retransmits corrupted frames |
| Hamming Code | Detects and corrects single-bit errors |
| Forward Error Correction (FEC) | Corrects errors without retransmission |

Debugging Steps

Step 1: Monitor Network Errors

Check:

- Frame Error Rate

- Packet Loss

- CRC Errors

Step 2: Inspect Physical Media

Check:

- Damaged cables

- Loose connectors

- Electrical interference

Step 3: Verify Error Detection

Ensure:

- CRC is enabled.

- Checksum is working correctly.

Step 4: Analyze Retransmissions

Monitor:

- Number of retransmitted frames

- Response time

- Congestion level

Step 5: Verify Database

Compare:

- Physical inventory

- Warehouse database

Correct inconsistent records.

Optimization Strategies

1. Use CRC for Error Detection

Advantages:

- Detects burst errors

- High accuracy

- Reliable transmission

2. Implement Hamming Code

Benefits:

- Detects errors

- Corrects single-bit errors

- Reduces retransmissions

3. Use Automatic Repeat Request (ARQ)

- Retransmit corrupted frames.

- Ensure accurate data delivery.

4. Reduce Electrical Interference

- Use shielded cables.

- Improve cable management.

- Keep network cables away from power lines.

5. Upgrade Network Infrastructure

- Replace damaged switches.

- Use high-quality Ethernet cables.

- Increase switch capacity.

6. Reduce Network Congestion

- Upgrade bandwidth.

- Prioritize critical warehouse traffic.

- Balance network load.

7. Continuous Monitoring

Monitor:

- CRC errors

- Packet loss

- Delay

- Throughput

- Retransmission count

Optimized Warehouse Network

Barcode      RFID      Warehouse PC

│          │             │

└────┬─────┴──────┬──────┘

│            │

┌──────▼────────────▼───────┐

│ Managed Network Switch    │

│ CRC + QoS + Error Monitor │

└──────────┬────────────────┘

│

Error Detection (CRC)

│

Error Correction (Hamming/ARQ)

│

┌────────▼────────┐

│ Central Server  │

│ Inventory DB    │

└─────────────────┘

✓ Reliable Data Transmission

✓ Accurate Inventory Records

✓ Reduced Packet Loss

✓ Fast Order Processing

✓ Low Transmission Errors

Expected Results After Optimization

- Accurate barcode scanning.

- Correct inventory records.

- Reduced transmission errors.

- Reliable RFID communication.

- Faster order processing.

- Fewer retransmissions.

- Improved network reliability and efficiency.

- A leading online education platform conducts live classes and online examinations for more than 15,000 students across different locations. During examinations, the central server simultaneously sends question papers, images, videos, and answer submission acknowledgements to all students over the network. The server is equipped with a high-speed 10 Gbps connection, whereas many students access the system using slower broadband or mobile internet connections. During peak hours, students experience incomplete downloads, delayed page loading, duplicate packets, and repeated retransmissions. Some students receive the next set of data before their devices finish processing the previous one, causing receiver buffer overflow and application crashes. Network administrators also observe an excessive number of retransmissions, increased latency, reduced throughput, and poor bandwidth utilization. The current flow control mechanism is not effectively regulating the transmission rate between the sender and receivers. As the network engineer, you have been assigned to investigate the communication problems, identify the causes of inefficient flow control, debug the existing system, and recommend suitable optimization techniques to ensure reliable and efficient data transmission during high-traffic periods.

Answer : 5

Flow Control Analysis and Optimization

Network Diagram

Online Education Platform

┌────────────────────────┐

│ Central Server (10 Gbps)│

│ Questions • Videos      │

│ Images • Acknowledgment │

└────────────┬────────────┘

│

High-Speed Network

│

┌─────────▼─────────┐

│   Internet/Router │

└─────────┬─────────┘

│

┌──────────┬──────────┼──────────┬──────────┐

│          │          │          │

Student 1  Student 2  Student 3  Student N

(100 Mbps) (50 Mbps)  (20 Mbps)  (Mobile)

│          │          │          │

Buffer Full  Slow Link  Packet Loss  Delay

Problems:

✖ Receiver Buffer Overflow

✖ Duplicate Packets

✖ Retransmissions

✖ Increased Latency

✖ Low Throughput

Faults Identified

1. Sender Transmits Too Fast

- Server has 10 Gbps speed.

- Students have slower Internet connections.

- Data reaches receivers faster than they can process.

2. Receiver Buffer Overflow

- Receiver memory becomes full.

- Incoming packets are dropped.

- Applications may crash.

3. Excessive Retransmissions

- Lost packets are retransmitted.

- Increases network traffic.

- Reduces performance.

4. Duplicate Packets

- Retransmitted packets may arrive again.

- Causes unnecessary processing.

5. Poor Flow Control

- Sender does not adjust its transmission speed.

- Leads to congestion and bandwidth wastage.

Performance Issues

Online Examinations

- Incomplete question paper download

- Delayed answer submission

Live Classes

- Video buffering

- Audio delay

Students

- Slow page loading

- Application crashes

Network

- High latency

- Reduced throughput

- Poor bandwidth utilization

Flow Control Concept

Flow control ensures that the sender transmits data only as fast as the receiver can process it, preventing receiver buffer overflow.

Sender

Receiver

Acknowledgments

Received packets

·····

TCP detects the lost third packet and retransmits it, so all five packets arrive.

Protocol

TCPUDP

TCPUDP

Packet loss

No lossDrop packet 3

No lossDrop packet 3

Debugging Steps

Step 1: Monitor Network Performance

Check:

- Packet loss

- Throughput

- Latency

- Retransmission count

Step 2: Check Receiver Buffer

Monitor:

- Buffer size

- Buffer occupancy

- Overflow events

Step 3: Verify Flow Control

Ensure:

- Sender waits for receiver acknowledgements.

- Transmission rate matches receiver capacity.

Step 4: Analyze Retransmissions

Identify:

- Duplicate packets

- Timeout events

- Congestion

Step 5: Test Network Capacity

Measure:

- Available bandwidth

- Link utilization

- Processing speed of student devices

Optimization Techniques

1. Sliding Window Flow Control  (Best Solution)

- Sender transmits only a limited number of packets.

- Waits for acknowledgements before sending more.

- Prevents buffer overflow.

2. TCP Flow Control

Use TCP features:

- Acknowledgements (ACK)

- Window Size

- Congestion Control

Benefits:

- Reliable transmission

- Reduced packet loss

3. Increase Receiver Buffer Size

- Allows more packets to be stored.

- Reduces overflow during peak traffic.

4. Traffic Scheduling

Give higher priority to:

- Question papers

- Answer submissions

Lower priority:

- Videos

- Images

5. Congestion Control

- Reduce transmission rate during congestion.

- Avoid excessive retransmissions.

6. Content Distribution

Use:

- Multiple application servers

- Content Delivery Network (CDN)

Benefits:

- Distributes traffic.

- Reduces load on the central server.

7. Continuous Monitoring

Monitor:

- Throughput

- Latency

- Packet loss

- Buffer usage

- Retransmissions

Optimized Network

Central Server (10 Gbps)

│

Sliding Window + TCP

│

Congestion Control

│

┌──────────────┼──────────────┐

│              │              │

Student 1      Student 2      Student N

ACK Window     ACK Window     ACK Window

│              │              │

Receive Data    Receive Data   Receive Data

Without         Without        Without

Overflow        Overflow       Overflow

✓ Controlled Transmission Rate

✓ No Buffer Overflow

✓ Reduced Retransmissions

✓ Better Throughput

✓ Efficient Bandwidth Usage

Expected Results After Optimization

- Complete and faster download of question papers.

- Smooth live video classes.

- Reliable answer submission.

- Reduced packet loss and retransmissions.

- No receiver buffer overflow.

- Lower latency.

- Higher throughput.

- Better bandwidth utilization.

- Stable performance for all 15,000+ students.